AirSense

Indoor Air Quality Monitoring & Prediction System

ESP32  ·  Air Quality Sensor  ·  Supabase  ·  ML Prediction

Technical Documentation  —  v1.0

SECTION 1

# Project Overview

AirSense is an end-to-end IoT system designed to continuously monitor indoor air quality, persist readings to a cloud database, and produce short-term air quality forecasts using historical trends. The system combines the low-cost ESP32 microcontroller with a multi-gas air quality sensor, Supabase as the cloud backend, and a lightweight machine learning pipeline built on two weeks of collected data.

## Objectives

- Continuously capture air quality metrics (PM2.5, CO2, TVOC, humidity, temperature) from a sensor attached to an ESP32.

- Transmit sensor readings over Wi-Fi to a Supabase PostgreSQL database.

- Accumulate a minimum of 14 days of historical data per monitored room.

- Use the historical dataset to train a time-series prediction model and forecast near-future air quality.

- Expose prediction results through Supabase Edge Functions so any client (dashboard, mobile app) can consume them.

## High-Level Architecture

The system is divided into three distinct layers:

| Layer | Components | Responsibility |
| --- | --- | --- |
| Edge / Device | ESP32 + MQ135 / BME680 / SGP30 | Sensor sampling, Wi-Fi transmission, deep-sleep power management |
| Cloud / Storage | Supabase (PostgreSQL + Storage + Auth) | Receive readings via REST, persist data, enforce row-level security |
| Analytics / ML | Python (scikit-learn / statsmodels) + Edge Functions | Train forecasting model on 14-day window, serve predictions |

SECTION 2

# Hardware Setup

## Component List

| Component | Details / Notes |
| --- | --- |
| ESP32 Dev Board | ESP32-WROOM-32 or ESP32-S3; 3.3 V logic, dual-core 240 MHz, built-in Wi-Fi & BLE |
| Air Quality Sensor | MQ135 (TVOC/NH3/CO2 analogue) or SGP30 (I2C eCO2 + TVOC) — see note below |
| Particulate Sensor | PMS5003 or SDS011 via UART for PM1.0 / PM2.5 / PM10 readings |
| Temp & Humidity | BME680 (I2C) — also provides barometric pressure and gas resistance |
| Power Supply | 5 V / 1 A USB adapter or LiPo + TP4056 charger module for portable use |
| Enclosure | 3D-printed or off-the-shelf ABS box; drill vent holes for airflow |
| Misc | Breadboard, jumper wires, 10 kΩ pull-up resistors for I2C lines |

Sensor Selection Note MQ135 is analogue and requires an ADC calibration step. Use a 12-bit ADC channel on the ESP32. SGP30 communicates over I2C and returns calibrated eCO2 (400–60,000 ppm) and TVOC values — preferred for accuracy. For a combined solution use: SGP30 (gas) + PMS5003 (particulates) + BME680 (temperature / humidity).

## Wiring Diagram

Connect the sensors to the ESP32 using the following pin mapping:

| Signal | ESP32 GPIO |
| --- | --- |
| I2C SDA (SGP30, BME680) | GPIO 21 |
| I2C SCL (SGP30, BME680) | GPIO 22 |
| PMS5003 TX → ESP RX | GPIO 16 (UART2 RX) |
| PMS5003 RX → ESP TX | GPIO 17 (UART2 TX) |
| MQ135 AOUT (if used) | GPIO 34 (ADC1_CH6, input only) |
| 3.3 V power rail | 3V3 pin |
| Ground | GND pin |

I2C Pull-up Resistors Place 4.7 kΩ resistors between SDA → 3.3V and SCL → 3.3V if your breakout boards do not already include them. Missing pull-ups cause intermittent I2C hangs.

SECTION 3

# ESP32 Firmware

## Development Environment

- IDE: Arduino IDE 2.x or PlatformIO (VS Code extension recommended)

- Board package: ESP32 Arduino Core v2.0+ (Espressif)

- Required libraries: ArduinoJson, Adafruit BME680, SparkFun SGP30, PMS Library, WiFi, HTTPClient

## Firmware Logic Flow

- Boot and connect to Wi-Fi using stored credentials.

- Initialize I2C bus and all sensor objects.

- Enter main loop: read sensor data every 60 seconds.

- Build a JSON payload containing room ID, timestamp, and all metric values.

- POST the payload to the Supabase REST API endpoint using HTTPS.

- On success, blink LED green; on failure, log error and retry after 10 s.

- After 10 consecutive successful readings, enter light-sleep for power saving.

## Sample Firmware Sketch

#include <WiFi.h> #include <HTTPClient.h> #include <ArduinoJson.h> #include <Adafruit_BME680.h> #include "SparkFun_SGP30_Arduino_Library.h" const char* SSID = "YOUR_WIFI_SSID"; const char* WIFI_PASS = "YOUR_WIFI_PASSWORD"; const char* SUPABASE_URL = "https://<project>.supabase.co/rest/v1/air_readings"; const char* ANON_KEY = "YOUR_SUPABASE_ANON_KEY"; const char* ROOM_ID = "living_room"; Adafruit_BME680 bme; SGP30 sgp; void setup() { Serial.begin(115200); WiFi.begin(SSID, WIFI_PASS); while (WiFi.status() != WL_CONNECTED) delay(500); bme.begin(); sgp.begin(); } void loop() { bme.performReading(); sgp.measureAirQuality(); StaticJsonDocument<256> doc; doc["room_id"] = ROOM_ID; doc["temperature"] = bme.temperature; doc["humidity"] = bme.humidity; doc["pressure"] = bme.pressure / 100.0; doc["eco2"] = sgp.CO2; doc["tvoc"] = sgp.TVOC; String body; serializeJson(doc, body); HTTPClient http; http.begin(SUPABASE_URL); http.addHeader("Content-Type", "application/json"); http.addHeader("apikey", ANON_KEY); http.addHeader("Authorization", String("Bearer ") + ANON_KEY); http.addHeader("Prefer", "return=minimal"); int code = http.POST(body); Serial.printf("HTTP %d\n", code); http.end(); delay(60000); // sample every 60 seconds }

## Power Management

For battery-powered deployments enable ESP32 light sleep between samples:

esp_sleep_enable_timer_wakeup(55 * 1000000ULL); // wake 5 s before next sample esp_light_sleep_start();

SECTION 4

# Supabase Backend

## Database Schema

Create the following table in your Supabase SQL editor:

CREATE TABLE air_readings ( id BIGSERIAL PRIMARY KEY, room_id TEXT NOT NULL, recorded_at TIMESTAMPTZ NOT NULL DEFAULT NOW(), temperature NUMERIC(5,2), -- °C humidity NUMERIC(5,2), -- %RH pressure NUMERIC(7,2), -- hPa eco2 INTEGER, -- ppm tvoc INTEGER, -- ppb pm25 NUMERIC(6,2), -- µg/m³ aqi_score NUMERIC(5,1) -- computed AQI (optional) ); -- Index for fast time-range queries per room CREATE INDEX idx_air_readings_room_time ON air_readings (room_id, recorded_at DESC);

## Row-Level Security (RLS)

Enable RLS so only authenticated devices can insert data:

ALTER TABLE air_readings ENABLE ROW LEVEL SECURITY; -- Allow inserts from the anon key (device uses anon key) CREATE POLICY "devices can insert" ON air_readings FOR INSERT WITH CHECK (true); -- Allow authenticated users to read all data CREATE POLICY "auth users can read" ON air_readings FOR SELECT USING (auth.role() = 'authenticated');

## REST API Endpoint

Supabase auto-generates a REST API. The device POSTs to:

POST https://<project-ref>.supabase.co/rest/v1/air_readings Headers: apikey: <anon-public-key> Authorization: Bearer <anon-public-key> Content-Type: application/json Prefer: return=minimal Body (JSON): { "room_id": "living_room", "temperature": 26.4, "humidity": 58.2, "pressure": 1012.3, "eco2": 820, "tvoc": 45 }

## Data Retention Policy

To avoid unbounded growth, create a scheduled job (pg_cron) that purges data older than 90 days while keeping daily aggregates:

-- Enable pg_cron extension (Supabase dashboard → Extensions) SELECT cron.schedule('purge-old-readings', '0 3 * * *', $$DELETE FROM air_readings WHERE recorded_at < NOW() - INTERVAL '90 days'$$);

SECTION 5

# Data Collection & Preparation

## Collection Phase (14 Days)

The prediction model requires a minimum of two weeks of continuous readings. During this phase:

- Ensure the sensor samples at a consistent interval — 60 seconds is recommended for granularity without excessive data volume.

- Do not move or cover the sensor during the baseline period, as positional changes introduce distribution shift.

- Record the room occupancy schedule (work hours, weekend patterns) as metadata — this significantly improves model accuracy.

- After 14 days you will have approximately 20,160 readings per sensor channel.

## Expected Data Volume

| Parameter | Value |
| --- | --- |
| Sampling interval | 60 seconds |
| Readings per day | 1,440 |
| Readings per 14 days | ~20,160 |
| Bytes per reading (JSON) | ~180 B |
| Total raw data (14 days) | ~3.5 MB |
| Supabase free-tier row limit | 500 MB — well within budget |

## Feature Engineering

Before training, extract the following features from the raw timestamps and readings:

- Hour of day (0–23) — captures circadian patterns.

- Day of week (0–6) — captures weekday vs. weekend occupancy.

- Rolling mean and standard deviation over 1 h, 4 h, and 24 h windows.

- Lag features: t-1, t-5, t-60 (1 min, 5 min, 60 min prior readings).

- AQI score derived from eCO2 and TVOC using EPA breakpoint formula.

SECTION 6

# Prediction Model

## Model Selection

Given the periodic nature of indoor air quality and the modest dataset size (~20 K rows), two models are recommended:

| Model | Strengths | Limitations |
| --- | --- | --- |
| SARIMA | Explicit seasonal decomposition, interpretable, no training GPU required | Manual parameter tuning (p,d,q)(P,D,Q,s), assumes stationarity |
| Random Forest Regressor | Handles non-linearity, works with engineered lag features, fast inference | No explicit time awareness, needs careful feature construction |

Start with Random Forest for simplicity; graduate to SARIMA if you need confidence intervals on predictions.

## Training Script (Python)

import pandas as pd from supabase import create_client from sklearn.ensemble import RandomForestRegressor from sklearn.model_selection import train_test_split from sklearn.metrics import mean_absolute_error import joblib SUPABASE_URL = "https://<project>.supabase.co" SUPABASE_KEY = "<service-role-key>" ROOM_ID = "living_room" # 1. Fetch last 14 days from Supabase sb = create_client(SUPABASE_URL, SUPABASE_KEY) resp = sb.table("air_readings") \ .select("*") \ .eq("room_id", ROOM_ID) \ .gte("recorded_at", "now()-interval '14 days'") \ .order("recorded_at") \ .execute() df = pd.DataFrame(resp.data) df["recorded_at"] = pd.to_datetime(df["recorded_at"]) df = df.set_index("recorded_at").sort_index() # 2. Feature engineering df["hour"] = df.index.hour df["weekday"] = df.index.weekday for col in ["eco2", "tvoc", "temperature", "humidity"]: df[f"{col}_lag1"] = df[col].shift(1) df[f"{col}_lag5"] = df[col].shift(5) df[f"{col}_roll1h"] = df[col].rolling(60).mean() df.dropna(inplace=True) # 3. Define target: eco2 value 30 min into future df["target_eco2"] = df["eco2"].shift(-30) df.dropna(inplace=True) FEATURES = [c for c in df.columns if c not in ["target_eco2", "id", "room_id"]] X = df[FEATURES] y = df["target_eco2"] # 4. Train / test split (time-aware) split = int(len(df) * 0.8) X_train, X_test = X.iloc[:split], X.iloc[split:] y_train, y_test = y.iloc[:split], y.iloc[split:] model = RandomForestRegressor(n_estimators=200, n_jobs=-1, random_state=42) model.fit(X_train, y_train) mae = mean_absolute_error(y_test, model.predict(X_test)) print(f"Test MAE: {mae:.2f} ppm") # 5. Persist model joblib.dump(model, "aqi_model.joblib") print("Model saved.")

## Serving Predictions via Supabase Edge Function

Deploy a Deno Edge Function that loads the model (or calls a Python microservice) and returns the forecast:

// supabase/functions/predict-aqi/index.ts import { serve } from 'https://deno.land/std@0.168.0/http/server.ts' import { createClient } from 'https://esm.sh/@supabase/supabase-js@2' serve(async (req) => { const { room_id } = await req.json() // Fetch last 60 readings for the room const sb = createClient(Deno.env.get("SUPABASE_URL")!, Deno.env.get("SUPABASE_SERVICE_KEY")!) const { data } = await sb .from("air_readings") .select("eco2, tvoc, temperature, humidity, recorded_at") .eq("room_id", room_id) .order("recorded_at", { ascending: false }) .limit(60) // Call Python prediction microservice const mlResp = await fetch(Deno.env.get("ML_SERVICE_URL")!, { method: "POST", headers: { "Content-Type": "application/json" }, body: JSON.stringify({ readings: data }), }) const prediction = await mlResp.json() return new Response(JSON.stringify(prediction), { headers: { "Content-Type": "application/json" }, }) })

SECTION 7

# Deployment & Operations

## Deployment Checklist

- Flash firmware to ESP32 and verify serial output shows successful Wi-Fi connection and HTTP 201 responses.

- Mount the sensor enclosure at breathing height (1.0–1.5 m from floor), away from windows and HVAC vents.

- Confirm rows appearing in Supabase Table Editor within the first 5 minutes of power-on.

- Wait 14 days for baseline data collection before running the training script.

- Train the model, evaluate MAE on the held-out set, and deploy the Edge Function.

- Set up Supabase Cron to retrain the model weekly with fresh data.

## Monitoring & Alerts

- Create a Supabase Database Function that fires a webhook when eCO2 exceeds 1,500 ppm.

- Use Supabase Realtime to push live readings to a web dashboard via WebSocket.

- Set up a pg_cron job to email a daily summary (min/max/avg per room).

## AQI Reference Thresholds

| eCO2 Level (ppm) | Indoor Air Quality Category |
| --- | --- |
| 400–600 | Excellent — fresh outdoor air level |
| 600–1,000 | Good — normal occupied spaces |
| 1,000–1,500 | Fair — ventilate soon |
| 1,500–2,000 | Poor — open windows immediately |
| > 2,000 | Very Poor — vacate room, check HVAC |

SECTION 8

# Troubleshooting

| Symptom | Likely Cause & Fix |
| --- | --- |
| HTTP 401 from Supabase | Wrong or expired anon key — regenerate in Supabase dashboard → Settings → API |
| I2C sensor returns 0x00 | Missing pull-up resistors or wrong SDA/SCL pin assignment — verify wiring |
| Wi-Fi connects but POST fails | Check HTTPS certificate — use WiFiClientSecure with correct root CA for Supabase |
| SGP30 eCO2 stuck at 400 | Sensor needs 15-minute warm-up baseline run before calibrated readings stabilize |
| Model MAE > 200 ppm | Insufficient data, outlier readings, or missing lag features — check for data gaps and add more lag columns |
| Edge Function cold-start timeout | Pre-warm by calling the function on a schedule; or migrate inference to a persistent Python service |
| Battery drains in < 2 days | Increase sleep interval to 5 min for non-critical monitoring; disable BLE radio at boot |

SECTION 9

# Future Enhancements

- OTA firmware updates — use ESP32 Arduino OTA library to push new firmware from Supabase Storage without physical access.

- Multi-room support — extend the schema with a rooms table and a foreign-key relationship; train one model per room or a multi-task model.

- Anomaly detection — add an Isolation Forest layer to flag unusual spikes that may indicate gas leaks.

- Mobile dashboard — build a Flutter or React Native app that consumes the Supabase Realtime channel and renders live AQI gauges.

- Federated learning — when deploying in multiple buildings, aggregate model updates without sharing raw data for privacy preservation.

APPENDIX

# Appendix

## A. Useful Supabase CLI Commands

# Login and link project supabase login supabase link --project-ref <project-ref> # Run migrations locally supabase db push # Deploy edge function supabase functions deploy predict-aqi # Inspect logs supabase functions logs predict-aqi

## B. Python Environment Setup

python -m venv .venv source .venv/bin/activate # Windows: .venv\Scripts\activate pip install supabase pandas scikit-learn joblib statsmodels

## C. References

- Supabase Documentation — https://supabase.com/docs

- ESP32 Arduino Core — https://github.com/espressif/arduino-esp32

- SparkFun SGP30 Library — https://github.com/sparkfun/SparkFun_SGP30_Arduino_Library

- Adafruit BME680 Library — https://github.com/adafruit/Adafruit_BME680

- PMS5003 Library — https://github.com/fu-hsi/PMS

- scikit-learn RandomForestRegressor — https://scikit-learn.org

— End of Document —